KIRIBATI SHIP REGISTRY

10 Anson Road, #25-14, International Plaza, Singapore 079903

Tel: +65 6225 0555 Fax: +65 6225 0550 Email: info@kiribaship.com

Website: https://kiribaship.com

FISHING VESSEL FLAG STATE INSPECTION REPORT

(FORM FSIFV)

Notes:

- This form is to be completed by the FSI Inspector with written comments where appropriate to document the vessel’s condition and crew competence. - Any deficiencies that are issued shall be listed on the last section “DEFICIENCIES AND CORRECTIVE ACTION”

| Vessel Name | Type of Vessel | IMO Number | Official Number | Gross Tonnage |  | Year Built |
| --- | --- | --- | --- | --- | --- | --- |
| Date / Time Inspection Started |  | Date / Time Inspection Ended |  | Country / Port of Inspection |  |  |
| Inspection Type Annual Follow up Mandatory FSI Physical or Remote |  |  |  |  |  |  |
| Name of Company Rep (if onboard) |  | Contact details (address/phone/email) of Company Rep (if onboard) |  |  |  |  |
| Name of FSI Inspector |  | Company Name of FSI Inspector |  |  | FSI Inspector ID Card No. |  |

# CHAPTER I DOCUMENTS AND SURVEYS

Confirm applicable Flag documents and survey dates. Treaty certificates apply only where legally applicable.

| CTA ref. | Check satisfactory | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |
| I/3–4 | Flag exemptions and alternatives/equivalent available; conditions followed. |  |  |  |  |
| I/9(1)(e) | Bottom inspections: two in five years; gap ≤36 months. Any extension approved. |  |  |  |  |

| I/11(3) | Exemption certificate available where required; linked to the safety certificate. (to the non-statutory SoC, where applicable; approval reference) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I/13 | Safety certificate valid (normally ≤5 years). Exemptions, extensions and endorsements valid. |  |  |  |  |

# CHAPTER II HULL AND WATERTIGHT CLOSURES

New vessels, unless Flag requirements extend application. Check approved plans and onboard condition.

| CTA ref. | Check satisfactory | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |
| II/1(1)–(3) | Hull, decks, bulkheads, where required, match approved plans and are sound. |  |  |  |  |
| II/1(4)–(7) | Collision bulkhead, penetrations and valves intact. Required double bottom fitted on ≥75 m vessels, where practicable. |  |  |  |  |
| II/2 | Watertight doors close correctly. Local/remote controls, indicators and closed-at-sea notices checked; 15° sliding-door capability documented. |  |  |  |  |
| II/3 | External openings secure. Stern-trawler fish flaps operate from a position with clear view. |  |  |  |  |
| II/4(1) | Weathertight door gaskets and clamps sound; doors operate from both sides. Approved freezer-room exception has trapped-person alarm. |  |  |  |  |
| II/4(2) | Door sills: working deck ≥600 mm; superstructure deck ≥300 mm. Any permitted reduction approved. |  |  |  |  |
| II/6 | Other hatch covers, gaskets, clamps and coamings sound. Reduced/omitted coamings and quick closing approved. |  |  |  |  |
| II/7–8 | Machinery casings, companionways, manholes and scuttles close securely; covers permanently attached where required. |  |  |  |  |
| II/9 | Ventilator coamings and closures sound; heights comply. Any omitted closure or alternative approved. |  |  |  |  |
| II/10 | Air pipes and attached closures sound. Heights: working deck ≥760 mm; superstructure ≥450 mm, unless approved lower. |  |  |  |  |
| II/11 | Tank/bilge sounding arrangements work. Closures attached; below-deck sounding pipes have self-closing devices. |  |  |  |  |
| II/12 | Windows, sidescuttles and deadlights sound. Sills ≥500 mm above deepest operating waterline; sidescuttles below 1,000 mm fixed. Wheelhouse safety glass fitted. |  |  |  |  |
| II/13 | Sea-inlet/discharge valves, non-return devices and indicators work; pipes and shell fittings sound. |  |  |  |  |
| II/14 | Freeing ports clear and match approved arrangement. Fishing gear and poundboards do not block drainage. |  |  |  |  |
| II/14(7) | In icing areas, freeing-port covers/protection can be removed easily. |  |  |  |  |
| II/15 | Anchors, chains, wires, stoppers, windlass and mooring gear sound and ready. |  |  |  |  |

# CHAPTER III STABILITY AND LOADING

New vessels, unless Flag requirements extend application. Check booklet first, then actual loading.

| CTA ref. | Check satisfactory | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |
| III/10 | Current approved stability booklet onboard; skipper understands loading limits and warnings. |  |  |  |  |
| III/9 | Inclining/lightship report available. Weight changes assessed; re-inclining completed where required. |  |  |  |  |
| III/1–2 | Approved stability calculations cover actual fishing and loading conditions; any departure approved. |  |  |  |  |
| III/7(1)–(2) | Approved loading cases cover departure, full catch, low stores and worst stability conditions. |  |  |  |  |
| III/7(3) | Actual loading follows approved limits for deck catch, wet nets, ballast and free surfaces. |  |  |  |  |
| III/4 | Stability information covers fishing-gear loads and fishing method. |  |  |  |  |
| III/2(2)–(3) | Ballast and roll-limiting arrangements match the approved stability basis. |  |  |  |  |
| III/3 | Fish-hold flooding angle ≥20°, or approved flooded-hold calculations available. |  |  |  |  |
| III/11 | Catch secured; fish-hold divisions sound and match approved arrangement. |  |  |  |  |
| III/14 | ≥100 m and ≥100 persons: approved subdivision and damage-stability calculations available. |  |  |  |  |

# CHAPTER IV MACHINERY AND ELECTRICAL SYSTEMS

New vessels ≥45 m, subject to Flag application. Unattended machinery checks apply only where fitted/approved.

| CTA ref. | Check satisfactory | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |
| IV/3 | Machinery and fish-processing equipment maintained. Guards, hot-surface protection, access and ventilation satisfactory. |  |  |  |  |
| IV/3(3)–(5) | Propulsion recovery and dead-ship starting arrangements verified. Required inclination and vibration capability documented. |  |  |  |  |
| IV/4 | Engine controls, pressure relief, alarms and protective shutdowns work. Any override/exemption approved. |  |  |  |  |
| IV/5 | Astern controls work; required astern/stopping performance records available. |  |  |  |  |
| IV/6 | Boiler safety devices work. Unattended oil-fired boiler alarms/fuel trips cover low water, air failure and flame failure Safety valve of adequate capacity, Unattended oil-fired boiler safety devices test & alarm, safety arrangement of steam boiler. |  |  |  |  |
| IV/7–8 | Two bridge–engine communication means work, including telegraph. Remote/local controls, station transfer and independent bridge emergency stop work. |  |  |  |  |
| IV/8 | Control-failure and low-starting-air alarms work; automatic-start limits and manual/local backup checked. |  |  |  |  |
| IV/9 | Starting-air pipes, relief devices, drains and backfire protection sound; oil contamination controlled. |  |  |  |  |
| IV/10(1) | Fuel flashpoint ≥60°C; emergency-generator fuel ≥43°C. Any permitted lower-flashpoint fuel approved with required precautions. |  |  |  |  |
| IV/10 | Oil tanks/pipes sound; level/pressure protection, spill control and ventilation satisfactory. No flammable oil in forepeak tanks. |  |  |  |  |
| IV/10(4) | Oil-tank quick-closing valves operate from outside the affected space. |  |  |  |  |
| IV/11 | Two independent power bilge pumps, or approved alternative, available; approved capacity confirmed. Test representative suction paths. Bilge ejector, direct bilge suction are working? |  |  |  |  |
| IV/11 | Bilge valves prevent flooding/cross-connection. Fish-processing spaces drain properly; collision-bulkhead closure works. Non-return bilge valve, valve of bilge piercing a collision-bulkhead with remote control, if valve is after side of bulkhead and ready accessible? |  |  |  |  |
| IV/12 | Machinery-space noise protection provided and effective. |  |  |  |  |
| IV/13(1)–(5) | Main/auxiliary steering work independently. Rudder indication, failure/overload/no-voltage alarms and motor indicators work. |  |  |  |  |
| IV/13(6)–(10) | Steering performance records: main 35° to opposite 30° ≤28 s; auxiliary 15° to opposite 15° ≤60 s, at prescribed conditions. |  |  |  |  |
| IV/13(8),(11) | Steering power restarts correctly. ≥75 m electric/electrohydraulic steering has two separated supply circuits. |  |  |  |  |
| IV/14 | ≥75 m: engineers’ alarm works and is audible in engineers’ accommodation. |  |  |  |  |
| IV/15(1)–(4) | Refrigeration plant/refrigerant approved; safety devices, drainage and refrigerant separation satisfactory. |  |  |  |  |
| IV/15(4)–(5) | Required refrigerant leak detection, ventilation and water spray work. Trapped-person alarm works; exit opens from inside. |  |  |  |  |
| IV/15(6)–(7) | Harmful refrigerants: two breathing-apparatus sets and spare cylinders available; emergency notices posted. |  |  |  |  |

| IV/16 | Main generators and required standby supply work; essential services and main/emergency lighting remain available. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| IV/17(1)–(3) | Independent emergency power protected from machinery-space fire; required services supplied for ≥3 hours. |  |  |  |  |
| IV/17(4)–(6) | Emergency fuel/starting or battery supply ready. Required automatic changeover, backup and battery ventilation satisfactory. Where a battery is the emergency source, its voltage must remain within ±12% of nominal throughout discharge. Automatic connection must immediately supply IV/17(2)(b) and (c) services; a manual auxiliary switch is required if automatic connection fails. |  |  |  |  |
| IV/17(7)–(8) | Test emergency starting/changeover and representative lights/loads. Required inclination capability documented. It can provide with full rated power in required direction, Test emergency starting/ changeover |  |  |  |  |
| IV/18 | Cables, earthing, insulation monitoring, switchboard guards/mats and circuit protection satisfactory. |  |  |  |  |
| IV/18 | Battery ventilation, hazardous-space electrical equipment, external isolators and lightning protection satisfactory. |  |  |  |  |

| IV/3(8)–(10) | Unattended machinery operation approved; routine inspection and test records current. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| IV/19 | Fuel-leak/overflow safeguards sound. Fire detection, audible/visual alarms and emergency supply work. |  |  |  |  |
| IV/19(7)–(10) | Required ≥2,500 kW engine oil-mist/bearing-temperature protection, fixed extinguishing and remote shutdowns work. ≥75 m: immediate fire-main water available. |  |  |  |  |
| IV/20 | Bilge high-level alarm works at watch position; sea/bilge valve controls accessible. |  |  |  |  |
| IV/21 | ≥75 m: bridge voice communication with machinery control and engineers’ accommodation works. |  |  |  |  |
| IV/22 | Fault alarms identify the fault, reach duty personnel and escalate if unattended; standby power works. |  |  |  |  |
| IV/23 | Automatic standby generation, load shedding and auxiliary changeover work; test records available. |  |  |  |  |
| IV/24 | Machinery safety shutdowns work. Any permitted override guarded and clearly indicated. |  |  |  |  |

# CHAPTER V FIRE SAFETY

New vessels ≥45 m. Part B: ≥60 m. Part C: 45–<60 m. Use the applicable approved arrangement.

| CTA ref. | Check satisfactory | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |
| V/1–2 | Fire-safety plans identify Part B/C, construction material, machinery spaces and applicable Method IF/IIF/IIIF. |  |  |  |  |
| V/3–8; V/28 | Fire divisions, insulation, linings and stair/lift protection intact; approved fire ratings maintained. |  |  |  |  |
| V/6–7; V/28 | Fire doors close correctly; self-closing and approved hold-open devices work. |  |  |  |  |
| V/8; V/11; V/28; V/31 | Draught stops, deck coverings and protected penetrations intact; exposed combustible insulation protected. |  |  |  |  |
| V/9; V/29 | Ventilation stops, dampers and inlet/outlet closures work from outside the served space. |  |  |  |  |
| V/9; V/29 | Galley grease trap clean; fan stop and required duct fire protection work. Flammable stores ventilated. |  |  |  |  |
| V/10; V/30 | Cooking/heating appliances safely secured and ventilated; required gas shutoffs work. No prohibited appliances. |  |  |  |  |
| V/12; V/32 | Gas cylinders and flammable stores secured in approved ventilated locations; electrical fittings and notices suitable. |  |  |  |  |
| V/13; V/33 | Required escape routes clear, separated and lead to open deck/survival craft. Any reduction approved. |  |  |  |  |
| V/15; V/34 | Required fire detectors, zone indications, fault alarms and main/emergency supplies work; coverage matches plan. |  |  |  |  |
| V/16 | Part B: high-fire-risk cargo spaces have approved fixed gas/equivalent protection. |  |  |  |  |

| V/17 | Part B: two fire pumps and independent alternative water supply ready. Required ≥75 m emergency pump independently driven. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| V/35 | Part C: required power fire pump ready; additional/emergency arrangements and independent controls provided where needed. |  |  |  |  |
| V/17–18; V/35–36 | Test fire-pump delivery; capacity meets approved basis. Main-pump hydrant pressure ≥0.25 N/mm². Record pressure. |  |  |  |  |
| V/17; V/35 | Pump relief/non-return valves and permitted changeover work; emergency pump/suction controls accessible outside main-pump space. |  |  |  |  |
| V/18; V/36 | Fire main, isolation valves and hydrants sound; required frost protection provided. |  |  |  |  |
| V/19 | Part B: two jets from separate hydrants reach accessible areas; one by a single hose length. |  |  |  |  |
| V/37 | Part C: one single-hose jet reaches accessible areas. Machinery entrance hydrant fitted; one hose per required hydrant plus spare. |  |  |  |  |
| V/19; V/37 | Fire hoses ≤20 m; couplings and dual-purpose nozzles sound, accessible and ready. |  |  |  |  |
| V/20–21; V/38–39 | Extinguishers serviced, correctly located, with spare charges. Accommodation/service/control spaces: ≥5 Part B; ≥3 Part C. |  |  |  |  |
| V/22; V/40 | Machinery-space fixed, foam/equivalent and portable fire protection matches approved requirements. |  |  |  |  |
| V/22; V/40 | Fixed-system release controls protected and accessible. Readiness verified safely; do not discharge extinguishing medium. |  |  |  |  |
| V/22(1)(b); V/40(1)(b) | No new halogenated-hydrocarbon extinguishing system installed on new or existing vessels. |  |  |  |  |
| V/23 | Part B: international shore connection, gasket, bolts and washers ready; usable on either side. |  |  |  |  |
| V/24; V/41 | Fireman’s outfits complete and ready. Part B: ≥2, widely separated. Part C: Flag-approved number/location. |  |  |  |  |
| V/25; V/42 | Current approved fire-control plan displayed, unless an applicable Flag dispensation is available. |  |  |  |  |
| V/26–27; V/43–44 | Fire equipment ready for immediate use; any alternative appliance/medium approved. |  |  |  |  |

# CHAPTER VI CREW PROTECTION

New vessels, unless Flag requirements extend application. Inspect decks, access and working areas.

| CTA ref. | Check satisfactory | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |
| VI/1(1) | Lifelines, ropes, shackles, eyebolts and cleats sound and ready. |  |  |  |  |
| VI/1(2)–(3) | Openings with sills/coamings <600 mm guarded; skylight bars ≤350 mm apart, unless exempted. |  |  |  |  |
| VI/1(4) | Decks and work/access areas have effective non-slip surfaces. |  |  |  |  |
| VI/2 | Covers cannot close accidentally; heavy escape covers open safely from both sides. Access hatches ≥600 ×600 mm or ≥600 mm diameter. |  |  |  |  |
| VI/3(1)–(2) | Exposed platforms protected by rails/bulwarks ≥1 m, unless approved lower. |  |  |  |  |
| VI/3(3) | Rails sound: lower gap ≤230 mm; other gaps ≤380 mm; stanchions ≤1.5 m apart. |  |  |  |  |
| VI/3(4) | Safe passage between accommodation, machinery and work spaces; gangways, lifelines and storm rails secure. |  |  |  |  |
| VI/3(5) | Stern-trawler ramp protected by doors/gates/nets; chain or barrier available when open. |  |  |  |  |
| VI/4 | Stairs/ladders sound, with handrails and non-slip treads. |  |  |  |  |

# CHAPTER VII LIFE SAVING EQUIPMENT

New vessels ≥45 m; VII/13–14 also apply to existing vessels ≥45 m. Check approved quantities and onboard readiness.

| CTA ref. | Check satisfactory | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |
| VII/3–4 | Life-saving appliance approvals match installed equipment; any novel alternative approved. |  |  |  |  |
| VII/5(1)–(5) | At least two survival craft; type, capacity and capacity on each side meet approved VII/5 arrangement. |  |  |  |  |
| VII/5(6)–(9) | Capacity after losing one craft adequate. Rescue boat/approved alternative provided; each boat marshals ≤9 rafts. |  |  |  |  |
| VII/6 | Survival craft/rescue boats ready; launching/recovery paths clear of gear and obstructions. |  |  |  |  |
| VII/6(2),(4) | Required davits or approved alternatives fitted where stowage/embarkation height exceeds 4.5 m. |  |  |  |  |
| VII/6(4)(f) | Liferaft painters, lashings and HRUs correctly rigged for float-free release, except permitted davit-launched arrangements. |  |  |  |  |
| VII/7 | Embarkation access, launch-area/water lighting and abandonment warning ready; craft protected from ship discharges. |  |  |  |  |
| VII/8; VII/24 | One approved lifejacket per person; locations marked. Jackets, lights and whistles sound. |  |  |  |  |
| VII/9; VII/25–26 | Required immersion suits/thermal aids available in suitable sizes; any warm-climate dispensation approved. Any dipensation? |  |  |  |  |
| VII/10(1)–(3) | Lifebuoys: ≥75 m, eight; <75 m, six. Half have lights; at least two lighted buoys have smoke signals. |  |  |  |  |
| VII/10(4)–(5) | One unlighted lifebuoy per side with line ≥30 m or twice stowage height, whichever greater. Buoys release easily. |  |  |  |  |
| VII/11; VII/28 | Line-throwing appliance approved and complete: ≥4 projectiles and ≥4 lines; instructions available. |  |  |  |  |
| VII/12; VII/29 | At least 12 approved rocket parachute flares; dates valid and stowage accessible/marked. |  |  |  |  |
| VII/13 | At least three approved portable two-way VHFs ready; any existing-vessel exception approved. |  |  |  |  |
| VII/14 | Radar transponder on each side, ready for transfer, or one in each survival craft. |  |  |  |  |
| VII/15 | Required reflective material intact on craft, lifejackets and lifebuoys. |  |  |  |  |
| VII/16(1)–(4) | Life-saving equipment ready; maintenance instructions/programme, spares and repair equipment available. |  |  |  |  |
| VII/16(3) | Falls turned end-for-end within 30 months; renewed within five years or earlier if damaged. Records available. |  |  |  |  |
| VII/16(5) | Weekly checks recorded: craft/launch gear, general alarm, and boat-engine run ≥3 minutes where temperature permits. |  |  |  |  |
| VII/16(6) | Monthly LSA/lifeboat-equipment checklist completed and logged. |  |  |  |  |
| VII/16(7) | Inflatable rafts/lifejackets serviced ≤12 months at approved station, unless extended by Flag. Permanent inflated-boat repairs by approved station. |  |  |  |  |
| VII/16(8)–(9) | Disposable HRUs within expiry; other HRUs serviced ≤12 months. Any 17/24-month extension for eligible equipment approved. |  |  |  |  |

| VII/17 | Lifeboat approval/capacity confirmed; hull, seats, drainage, rudder, painter, release and recovery fittings sound. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| VII/17(6)–(8) | Lifeboat engine/starting work; fuel/endurance confirmed. Equipment, food, water, first aid, tools and signals complete. |  |  |  |  |
| VII/18 | Partially enclosed boat: canopy, entrances, ventilation, visibility and oars satisfactory. |  |  |  |  |
| VII/19 | Totally enclosed boat: enclosure, hatches, belts and engine controls sound; self-righting arrangement approved. |  |  |  |  |
| VII/20 | Liferaft approval/capacity confirmed; container, canopy, boarding, painter, HRU/weak link and equipment pack satisfactory. |  |  |  |  |
| VII/21 | Inflatable raft service certificate valid; raft type, markings, container and applicable davit arrangement satisfactory. |  |  |  |  |
| VII/22 | Rigid raft: approval/capacity, buoyancy, boarding, stability, markings and condition satisfactory. |  |  |  |  |
| VII/23 | Rescue boat ready; hull/inflation, engine, steering and towing gear work. Recovery/marshalling capability confirmed. |  |  |  |  |
| VII/24–27 | Lifejackets, suits, thermal aids, lifebuoy lights, lines and smoke units sound; batteries/dates valid. |  |  |  |  |
| VII/29–31 | Survival-craft flares/smoke signals complete and within date; instructions legible. |  |  |  |  |
| VII/32 | Launch-gear approval/proof-test records available. Davits, winches, brakes, falls and releases sound. |  |  |  |  |
| VII/32 | Launching, lowering, recovery, limit stops and releases work; applicable free-fall/slide arrangements verified. |  |  |  |  |
| VII/32(7) | Embarkation ladders, handholds, steps and ropes sound, correctly sized and securely rigged. |  |  |  |  |

# CHAPTER VIII EMERGENCY DRILLS AND TRAINING

New and existing vessels ≥24 m. Check drill records and crew performance; apply approved relaxations only.

| CTA ref. | Check satisfactory | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |

| VIII/2(1) | General alarm works on required main/emergency supplies: ≥7 short blasts followed by one long blast. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| VIII/2(2)–(9) | Muster list posted and current; duties, substitutes, alarm and abandon-ship order clear. Any <45 m relaxation by Flag may be approved. |  |  |  |  |
| VIII/3(1)(a) | Each crew member attends monthly fire/abandon-ship drills; approved <45 m interval never exceeds three months. |  |  |  |  |
| VIII/3(1)(a) | Drills within 24 hours of departure if >25% missed previous muster, unless accepted equivalent applies. |  |  |  |  |
| VIII/3(1)(b),(d),(e),(j) | Abandon-ship drill: muster, duties, lifejackets, craft preparation/lowering, engines and raft davits checked, as applicable. |  |  |  |  |
| VIII/3(1)(c),(j) | Fire drill: duties, pump/jets, outfits, communications, closures/dampers, escape and abandonment preparation demonstrated. |  |  |  |  |
| VIII/3(1)(f)–(h) | Lifeboats launched/manoeuvred every three months. Rescue boats monthly where practicable, at least three-monthly. Headway launching safely supervised. |  |  |  |  |
| VIII/3(1)(i) | Emergency muster/launch lighting tested at each abandon-ship drill. |  |  |  |  |
| VIII/3(2) | LSA training within two weeks of joining; all LSA covered within two months. Davit-raft training ≤4 months apart. |  |  |  |  |
| VIII/3(3) | Drills/training logged; missed or partial drills explained. |  |  |  |  |
| VIII/3(4) | LSA training manual/approved audiovisual information available; any <45 m relaxation retains suitable safety information. |  |  |  |  |
| VIII/4 | Crew explain emergency duties, escape, firefighting, craft/radio use, man-overboard recovery, hypothermia and survival. |  |  |  |  |

# CHAPTER IX RADIOCOMMUNICATIONS

New and existing vessels ≥45 m. Apply approved sea areas. Use safe tests; never send a false distress alert.

| CTA ref. | Check satisfactory | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |
| IX/1–3 | Radio sea areas and any existing-system acceptance, deferral or exemption granted; required safety functions retained. |  |  |  |  |
| IX/4 | Required distress/safety functions available, including two independent ship-to-shore alerting means except permitted cases. |  |  |  |  |
| IX/5 | Radio installation protected; controls lit independently; station identity marked. Navigation VHF available at conning position. |  |  |  |  |
| IX/6(1)(a)–(b) | VHF voice channels, DSC channel 70 and continuous DSC watch work; bridge distress initiation available. |  |  |  |  |
| IX/6(1)(c)–(e) | Required 9 GHz transponder and route-appropriate NAVTEX/EGC or approved MSI alternative work. |  |  |  |  |
| IX/6(1)(f); IX/7(3) | Approved EPIRB ready; identity, battery, manual release and float-free arrangement checked. Use safe self-test. |  |  |  |  |
| IX/7 | A1: bridge distress alerting and VHF communications work; any VHF-EPIRB alternative approved. |  |  |  |  |
| IX/8 | A2: MF voice/DSC, DSC watch and independent non-MF distress means work. |  |  |  |  |
| IX/9 | A3: approved satellite-plus-MF or MF/HF arrangement works; watches and independent alerting verified. |  |  |  |  |
| IX/10 | A4: required MF/HF, watches and independent non-HF distress means work. |  |  |  |  |
| IX/11 | Required DSC/satellite watches and MSI reception maintained for sea area; crew know watch settings. |  |  |  |  |
| IX/12(1)–(5) | Main/emergency radio power and independent reserve supply work; simultaneous required loads supported. |  |  |  |  |
| IX/12(2) | Reserve duration meets IX/12(2): new ≥3 h; existing ≥6 h, or ≥3 h with compliant emergency power; ≥1 h only with compliant ≥6 h emergency supply. |  |  |  |  |
| IX/12(6)–(8) | Battery recharge ≤10 h; capacity checked ≤12 months. Battery condition/ventilation and navigation inputs on power failure satisfactory. |  |  |  |  |
| IX/13 | Radio approvals match installed equipment; any permitted older-equipment exception approved. |  |  |  |  |
| IX/14(1)–(5) | Radio manuals, tools and spares available; equipment accessible for maintenance/replacement. |  |  |  |  |
| IX/14(6)–(7) | Maintenance arrangements approved. A3/A4: ≥2 methods (duplication/shore/at-sea), unless Flag permits one. |  |  |  |  |
| IX/14(8) | Radio defects recorded; distress/safety capability and required corrective action assessed. |  |  |  |  |
| IX/15 | Qualified radio personnel onboard; one person responsible for distress communications. |  |  |  |  |
| IX/16 | Radio log contains required safety incidents and communications. |  |  |  |  |

CHAPTER X NAVIGATIONAL EQUIPMENT AND ARRANGEMENTS

New and existing vessels. Apply each item’s length/build-date limits and approved exemptions.

| X/2–3(1) | ≥24 m: required compass, heading display, compass-position communication and nearly 360° bearing means ready; any exemption approved. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| X/3(1)(b)–(c) | Compass adjusted; deviation table onboard. Required spare compass or permitted alternative available. |  |  |  |  |
| X/3(3)–(4) | Gyro/repeaters work: ≥45 m built from 1 Sep 1984; ≥75 m older vessels. Required bearing repeaters positioned correctly. |  |  |  |  |
| X/3(5) | Emergency steering has heading communication. ≥45 m built from 1 Feb 1992: required compass-reading arrangement available. |  |  |  |  |
| X/3(6)–(8) | ≥35 m: required 9 GHz radar and plotting work; any permitted exception approved. Fitted radar on smaller vessels accepted. |  |  |  |  |
| X/3(9)–(10) | Echo sounder fitted for X/3(9) categories (≥75 m before 25 May 1980; ≥45 m from 25 May 1990). <45 m: approved depth means. |  |  |  |  |
| X/3(11)–(12) | Required speed/distance and rudder/propeller/pitch/thruster indicators work; visible from conning position. Apply X/3 length/build-date limits. |  |  |  |  |
| X/3(14) | ≥75 m: radio direction finder or approved exemption/alternative available for intended voyages. |  |  |  |  |
| X/3(13),(16) | Navigation equipment approvals and condition satisfactory; defects assessed under X/3(13) and I/6. |  |  |  |  |
| X/4 | Current charts, sailing directions, lights lists, notices, tides and other publications cover the voyage. |  |  |  |  |
| X/5 | Daylight lamp and independent/portable battery supply work. ≥45 m: flags/pennants complete. Required International Code of Signals onboard. |  |  |  |  |
| X/6(1)(a)–(b) | New ≥45 m: forward view and blind sectors comply; gear does not obstruct view. Existing: comply where practicable. |  |  |  |  |
| X/6(1)(c)–(h) | New ≥45 m: horizon, conning, bridge-wing, steering and ship-side views meet approved arrangement. |  |  |  |  |
| X/6(1)(i),(3) | Applicable bridge windows give clear all-weather view; no tinted/polarised glass. Any unconventional arrangement accepted by Flag. |  |  |  |  |

# ADDITIONAL FLAG CHECKS MANNING AND CREW

Apply the vessel’s Flag requirements. This section is outside CTA Chapters I–X.

| Flag inspection check |  | Yes | No | N/A | Remarks / defect no. |
| --- | --- | --- | --- | --- | --- |
| 1.* | Crew list matches persons onboard; number within approved life-saving capacity. |  |  |  |  |
| 2.* | Manning meets the Flag safe manning document and restrictions. |  |  |  |  |
| 3.* | Skipper, watchkeepers, engineers and radio personnel hold valid required qualifications and Flag authorizations. |  |  |  |  |
| 4. | Crew medical fitness documents valid. |  |  |  |  |
| 5. | Watchkeeping and rest records satisfactory for navigation, machinery and fishing work. |  |  |  |  |
| 6.* | Crew understand emergency duties in a common language; new crew familiarized with alarms, exits and equipment. |  |  |  |  |
| 7* | Fishing winch emergency stops, guards, hydraulic systems and safe controls satisfactory; lifting gear examination and SWL records checked where required. |  |  |  |  |
| 8* | Enclosed-space entry, gas testing, isolation and hot-work procedures verified; crew familiar with applicable rescue arrangements. |  |  |  |  |
| 9* | P&I as per Marine Circular, Crew agreements, wages, repatriation, accommodation, sanitation, food, water, medical supplies and complaints arrangements checked under applicable Flag requirements |  |  |  |  |
| 10* | Previous FSI/PSC/RO findings and casualty-related repairs reviewed; closure evidence available and outstanding actions identified. |  |  |  |  |

# EQUIPMENT RECORD

Complete applicable items only. Enter quantities, locations, expiry/service dates and test results.

| Item / identification | Required / fitted | Capacity / location | Expiry / service date | Report / test evidence |
| --- | --- | --- | --- | --- |
| Survival craft / rescue boat |  |  |  |  |
| Lifejackets / immersion suits / thermal aids |  |  |  |  |
| Lifebuoys / lights / smoke / lines |  |  |  |  |
| Craft VHF / radar transponders |  |  |  |  |
| Distress flares / line-throwing appliance |  |  |  |  |
| Inflatable LSA / HRUs / launching falls |  |  |  |  |
| Fire pumps / extinguishing systems |  |  |  |  |
| Portable extinguishers / fireman’s outfits |  |  |  |  |
| Radio / EPIRB / reserve battery |  |  |  |  |
| Navigation / signalling equipment |  |  |  |  |

# DEFICIENCIES AND CORRECTIVE ACTION

Record each defect, Ref/item (as per CTA), Action required and due date. Notify RO and Port Authority if required.

| Ref. / item | Deficiency | Action required |  | Responsible / due | Closure / verified |
| --- | --- | --- | --- | --- | --- |
| Inspector name / signature / date |  |  | Skipper name / signature / date or Company Rep |  |  |

Skipper’s signature confirms receipt. This form does not issue a certificate or approve an exemption.